| Part #: | TBD |
| --- | --- |
| Description: | ADCP and GPS ANTENNA SETUP |

# Revision History:

| 4/15/2023 | Initial release |
| --- | --- |

# Documents:

|  |  | TBD |
| --- | --- | --- |

# Install TerraTerm, test GPS coms:

- Locate TerraTerm installer and move over to boat PC DOWNLOAD folder

- Run installer with defaults

- Start TerraTerm: use 4800 baud to start and confirm data coming in on correct port

# Install PocketMax, set GPS settings:

- Ensure wiring is correct and test power to GPS

- Locate PocketMax 4.0 installer

- Copy Desktop folder (has one msi and one exe) to boat PC DOWNLOAD\PocketMax folder

- Run installer on boat PC, accept defaults

- PocketMax4 icon should appear on desktop

- Locate and copy over V200 Settings file to PocketMax folder

- Start PocketMax4

- Select port, baud to 4800, click Connect

- When prompted, click Quick Config

- Once window fills, click File, Load Configuration File, select V200 Settings, click Open

- PocketMax4 should step through settings: when done click Close

- Click File, Save Settings

- Close PocketMax4, then power cycle the GPS

- Startup TerraTerm, confirm data looks good at 19200

# Check ADCP prior to install:

- Check order to see if ADCP is configured for RS-232 or RS-422

- If configured for RS-422, then remove top plate with 10mm wrench and socket head

- Locate the RS-232/422 switch, and set as shown below

- Replace top plate, tightening four bolts finger tight to start

- Use cross pattern to tighten until split washer flattens out

- Continue for another ¼ turn to compress the o-ring

- Use torque wrench to set final load to 5.6 Nm (50 in-lbs)

- See WorkHorse Operation Manual_Jan20.pdf: page 125 for switch, 98 for end cap

# Check ADCP:

- Start TerraTerm with baud at 9600

- Cycle power to ADCP, confirm characters appear

- If ADCP is already on, send break (Control, Send break) and confirm response

# Install RDI software, basic setup:

- Locate and copy over to _DOWNLOADS\RDI folder:

  - RDI Tools Setup June 2018.exe

  - VMDas Folder

- Install RDI Tools, use defaults

- Install VMDas, use defaults

- Start BBTalk (shortcut on Desktop)

- For Device select Work Horse, for COM select right port, click Next

- Leave default baud etc (9600, N, 8, 1)

- Click Finish to get terminal window

- Click the B to send a break and confirm coms

- At the > prompt, type: cb511 (sets buad to 19200, N, 8, 1)

- Should get CK and message saying saved

- Cycle power to ADCP, then restart the BBTalk at 19200 to confirm

- At prompt, type PS0 for system info

- type PA to init ADCP and perform pre-deployment tests: check output ??

- type PC1 to check beams out of water, rubbing faces

- type PT200 to run all onboard tests: can be out of water?

# Setup ADCP folder, test VMDas:

- Locate and copy _ADCP folder for customer to C:\_ADCP

- Start VMDas, then File, Collect Data

- Click Options, Load, then select the 300 Defaults.ini in the _ADCP folder

- Click Options, Edit Data Options

- Under Communications

  - Check ADCP port and NMEA 1 port

- Under ADCP Setup

  - Confirm ADCP Setup from File is set to correct ini (use Browse)

- Under Recording

  - Confirm Primary Path is set correctly (use Browse)

- Under NAV

  - Confirm both are set to NMEA1

- Under Averaging

  - Use 15 / 60 for bottom track and calibration

  - Use 60 / 300 for normal use

# Standard Startup and Run:

- Power up GPS and ADCP

- Start BBTalk

- For Device select Work Horse, for COM select right port, click Next

- Set baud 19200, N, 1, N, click Next, click Finish

- Click the B to send a break and confirm coms

- type PA to init ADCP and perform pre-deployment tests

- Close BBTalk, startup VMDas

- Click File, Collect Data, Play

- Confirm text appears in both text windows: ADCP and GPS verifying coms to both

- Click Stop when done

# Calibration of ADCP: still working on this

- Turn on bottom track settings or use appropriate setup TXT file

- Use the 15 / 60 settings for calibration

- Turn on Leader (LDR), Bottom Track (BOT) and Navigation (NAV) panes

- Compare BOT Direction to NAV Made Good Degrees

- Setting is in Transform tab: EA Heading alignment error

- Save INI file to store (may be automatic? – check)